# ReArm Candour

Product & Compliance Specification

| Field | Value |
| --- | --- |
| Document title | ReArm Candour — Product & Compliance Specification |
| Document ID | RST-SPEC-001 |
| Version | v0.1 |
| Classification | Internal — Pre-Certification Draft |
| Prepared by | [Name / role] |
| Date | 19 June 2026 |
| Status | Draft for internal audit review — not a certified compliance report |
| Distribution | Internal audit / compliance stakeholders |

# 1. Purpose & Scope

This document summarizes the ReArm Candour product concept, system architecture, and known compliance, safety, and data-protection considerations, to support internal audit and pre-certification review.

This is a planning and review document, not a certified compliance report. Formal regulatory testing, legal review, and sign-off by accredited test houses and qualified counsel are required before market release. Nothing in this document should be treated as a certification or legal determination.

# 2. Product Description & Intended Use

ReArm Candour is a two-part wireless add-on accessory for prescription or cosmetic eyeglasses that adds gaze (eye-tracking) data capture.

Components:

- Satellite sensor module — mounted at the nose bridge of the glasses.

- Mainboard companion device — worn or carried separately from the glasses.

- Charging case accessory — provides fast and ambient charging for the satellite module.

Intended use: to be formally defined by the product owner (for example: consumer accessibility aid, AR/gaming input device, or research data-collection tool). The intended use materially changes which regulatory regime applies — for instance, marketing claims that touch on diagnosis or treatment would trigger medical-device regulation.

Current working assumption: this is a consumer, non-medical accessory. This assumption must be confirmed and documented before proceeding, as it underpins the regulatory analysis in Section 4.

# 3. System Overview

| Component | Role | Notes |
| --- | --- | --- |
| Satellite (nose bridge) | Captures gaze data and extracts coordinates on-device | Designed to be minimal in size and weight |
| Mainboard | Receives, decrypts, and processes gaze data | Carried by the user or left stationary nearby |
| Charging case | Houses charging electronics for the satellite module | Accessory item, not part of the active data path |

Wireless link: short-range 2.4 GHz proprietary protocol, AES-128 encrypted, designed for in-home range (tens of metres). Only extracted X/Y gaze coordinates are transmitted — raw imagery is not sent wirelessly.

# 4. Regulatory & Standards Considerations

The table below identifies frameworks that are likely relevant given the product description. It is a starting point for due diligence, not a determination of applicability or compliance.

| Area | Likely applicable framework | Why it matters | Status |
| --- | --- | --- | --- |
| Radio emissions | UK: Radio Equipment Regs / UKCA; EU: RED / CE; US: FCC Part 15 | The 2.4 GHz radio is an intentional radiator and typically needs type approval | Not yet tested |
| RF exposure near the body | ICNIRP guidelines / SAR-style assessment | Device is worn on the head; near-body exposure limits can apply even at low transmit power | Not yet assessed |
| Battery safety | IEC 62133-2 (cell/pack safety); UN 38.3 (transport) | Uses a fast-charge LTO cell worn close to the face | Not yet tested |
| Electrical / thermal safety | IEC 62368-1 (or IEC 60950-1 legacy) | General consumer product electrical safety | Not yet assessed |
| Materials | RoHS; REACH | Standard requirement for electronics sold in the UK/EU | Not yet assessed |
| End-of-life | WEEE | Producer take-back and labelling obligations for electronic accessories | Not yet assessed |
| Data protection | UK GDPR / EU GDPR | Gaze data may qualify as biometric or special-category data | See Section 5 |

Engage an accredited test lab and qualified legal counsel to confirm which obligations actually apply before sale, particularly once the final intended use (Section 2) is locked in.

# 5. Data Privacy & Biometric Data Considerations

- Gaze-tracking data can reveal sensitive information about a person (attention patterns, and potentially health-related signals), and may be treated as biometric data under data protection law if it is used to identify an individual.

- A privacy-supportive design choice already in place: the satellite extracts X/Y coordinates on-chip and never transmits raw imagery, which limits what is exposed over the wireless link.

- Recommended before launch: a Data Protection Impact Assessment (DPIA), a documented data retention and use policy for any companion app or storage, and a clear justification if any gaze data leaves the device (for example, to an app or cloud service).

- Encryption: the link uses AES-128. The key-provisioning method (on-chip TRNG-generated session keys vs. a static shared key) needs to be finalized and verified in code review — “no hardcoded key” is currently a design intent, not yet a confirmed implementation.

# 6. Safety Risk Register

| Risk | Current design mitigation | Residual risk / open action |
| --- | --- | --- |
| Battery thermal event during fast charge | LTO chemistry tolerates high charge rates better than standard LiPo | Validate with cell-level abuse testing before production |
| RF exposure near eyes/face | Default low transmit power when the mainboard is nearby; power only increases when needed for range | Formal RF exposure assessment not yet performed |
| Mechanical detachment from glasses | Attachment method not yet finalized | Define attachment method and run mechanical fatigue testing |
| Small-parts / ingress hazard | Sealed enclosure intended | Confirm enclosure design and ingress protection rating |
| Heat against skin at the nose bridge | Low average current draw target (≈1.5 mA) keeps the module cool | Confirm with thermal testing once the gaze sensor is finalized |

# 7. Security Considerations

- Wireless link is encrypted using AES-128.

- No firmware-hardcoded keys is a stated design goal; this needs implementation and code review to confirm before audit sign-off.

- Firmware update path (over-the-air or wired) is not yet defined. A signed-firmware update mechanism is recommended to prevent unauthorized firmware on a body-worn device.

# 8. Quality & Reliability Notes

Headline claims that need formal validation before any audit sign-off:

- 48-hour runtime per charge — currently a calculated estimate, not a measured result.

- ≤10-minute full recharge — currently a calculated estimate, not a measured result.

Recommended test plan: battery cycle-life testing, real-world RF range testing across representative home layouts, and everyday-wear environmental testing (moisture/sweat exposure, drop testing for the nose-bridge module).

# 9. Open Items Before Audit Sign-Off

- Formally define the intended use statement (this affects regulatory classification — see Section 2).

- Complete a DPIA covering gaze-data handling.

- Finalize the gaze sensor selection and re-validate the power/thermal budget.

- Commission formal RF emissions and exposure testing once a working prototype exists.

- Commission battery safety testing (IEC 62133-2 / UN 38.3) on the chosen cell.

- Define and implement the firmware key-management and update-signing approach.

# 10. Revision History

| Version | Date | Author | Change description |
| --- | --- | --- | --- |
| v0.1 | 19 June 2026 | [Name] | Initial draft for internal audit review |

# 11. Approval

Prepared by:  ___________________________	Date:  ___________________

Reviewed by (technical):  ___________________________	Date:  ___________________

Reviewed by (compliance/legal):  ___________________________	Date:  ___________________

Approved by:  ___________________________	Date:  ___________________