# ReArm Candor

Engineering Specification — Working Draft

| Field | Value |
| --- | --- |
| Document title | ReArm Candor — Engineering Specification |
| Version | v0.3 (working draft — supersedes v0.2, “Candour”) |
| Status | Draft — architecture updated to live image streaming (no on-device storage); power budget recalculated; not bench-validated |
| Owner | Shemaiah Osuchukwu, Founder & CTO, ReArm |
| Last updated | 26 June 2026 |
| Distribution | Internal / maker use only — not for external release |

# 1. Product Overview

ReArm Candor is a smart-eyewear bridge module for prescription or cosmetic glasses that adds two capabilities: outward-facing stereo (RGB + IR-assisted) depth sensing of the environment, and inward-facing binocular gaze tracking of the wearer’s eyes — fused together to determine exactly what the wearer is looking at and how far away it is. A separate, less space-constrained companion mainboard (worn or carried nearby) handles heavier compute, the live ML image-processing/object-detection pipeline, and battery duty.

- Bridge module integrates cleanly into the glasses’ nose bridge with a premium, CNC-machined finish — no exposed screws or visible wiring.

- Front camera images (RGB and IR-assisted) stream live, continuously, to an ML pipeline for image processing and object detection — no image data is stored at any point in the pipeline.

- Inward gaze sensing must not use inward-facing cameras (privacy, power, and thickness) — use a flat IR photodiode array instead.

- Gaze vectors (inward) combine with the 3D scene map derived from the front camera stream (outward) to identify exactly what the user is looking at and how far away it is — vergence between the two gaze vectors gives fixation depth without extra depth hardware on the eye side.

- Wireless link to the mainboard must now carry a continuous live video stream, not just small coordinate packets — a major change from the original lightweight-payload design; see Section 9.

- All communication encrypted end-to-end.

ASSUMPTION: this revision assumes the ML image-processing/object-detection pipeline runs on the Mainboard companion device (which has the larger battery and compute budget), not on the bridge module itself or in the cloud, and that the rear binocular gaze array stays the low-power IR photodiode design from Section 4 rather than becoming camera-based. If either assumption is wrong — for example, if “ir images” refers to the eyes rather than the front scene, or if ML inference is meant to run on the bridge or off-device — the power and bandwidth numbers in Sections 7 and 9 would need to be reworked. Flag this so it can be corrected if needed.

SCOPE CHANGE: live, continuous video streaming for ML processing is a fundamentally different power profile from the original “send processed coordinates only” design principle. As shown in Section 7, this breaks the original 48-hour / ultra-tiny-module combination — something has to change (duty cycle, module size, or streaming approach). Read Section 7 before treating the original 48-hour and size targets as still both achievable.

# 2. System Architecture

| Component | Role | Notes |
| --- | --- | --- |
| Bridge Module (on the glasses) | Outward stereo depth sensing + inward binocular gaze tracking + on-board fusion compute | Replaces the original “Satellite” node; now houses meaningfully more silicon than originally scoped |
| Companion Mainboard (carried / placed) | Receives processed outputs; heavier logging/compute; larger battery | Role unchanged from v0.1 |
| Charging Case (accessory) | Reservoir battery, Qi receiver coil, pogo-pin dock | Concept unchanged; charge profile needs re-deriving once real bridge power draw is known |

Data flow:

The front stereo cameras build a 3D map of the scene. The rear photodiode arrays estimate each eye’s gaze vector, blink, pupil size, and eye openness, plus vergence (the convergence angle between the two gaze vectors, which gives fixation distance with no extra depth hardware on the eye side). The bridge’s on-board vision processor fuses the gaze rays with the 3D scene map to determine the object being looked at and its distance. Only this fused, compact result — not raw video and not raw photodiode traces — is intended to be sent over the wireless link to the mainboard, consistent with the original “coordinates, not video” design principle.

# 3. Bridge Module — Front-Facing Stack (outward, away from the eyes)

Per the reference render: a single horizontal bridge housing spans the nose, with two RGB/IR-assisted camera lenses, illumination elements, and a status indicator visible on the outward face.

- Stereo RGB/IR-assisted camera pair — captures the live scene continuously; images are streamed off the bridge module rather than processed there.

- IR illumination — for low-light scene capture (visible in the render; exact part TBD).

- IMU, and optionally a ToF sensor — for head motion/orientation, supporting the ML pipeline’s scene reconstruction.

- Lightweight on-board MCU — handles sensor control and stream packetization/compression only. Per this revision, the heavier on-board AI vision/fusion processor from v0.2 is no longer needed on the bridge module, since image processing and object detection now run on the mainboard’s ML pipeline (see Section 6).

- Status LED — visible in the render; recommend reserving this as an active-streaming/recording indicator (see the compliance specification’s bystander-privacy section).

OPEN ITEM: exact camera modules, illumination part numbers, IMU/ToF part, and the video compression/codec approach are not yet specified. The codec choice in particular drives the radio bandwidth and power figures in Sections 7 and 9.

# 4. Bridge Module — Rear-Facing Stack: the Gaze Array (inward, toward the eyes)

This replaces the previously open “gaze sensor” item from the original spec. Approach: a flat IR photodiode array per eye, with no inward-facing cameras, using differential reflectance off the sclera, iris, pupil, and eyelid to infer gaze (sometimes called photoscleral oculography).

## 4.1 Per-eye sensor geometry

- 16–32 IR photodiodes per eye, arranged as a 4×4 or 5×5 matrix, or as a simpler 4-node differential quadrant (top / bottom / left / right) around the eye perimeter.

- 2–4 IR LEDs per eye, 940 nm — invisible to the human retina, avoiding glare or distraction.

- Both sit behind a flush, smoked IR-transparent window so nothing is visible from outside the lens — matching the reference render’s intent of looking like sealed consumer electronics rather than a visible sensor grid.

## 4.2 Candidate components

| Component | Role | Package | Notes |
| --- | --- | --- | --- |
| Vishay VEMD2704 | IR photodiode (receiver) | 2.3×2.3 mm leadless SMD | Daylight-blocking filter, 830–950 nm, ≈70 ns response — fast enough for saccades |
| Everlight PD15-22C/TR8 | IR photodiode, lower-cost alternative | 1206 SMD | Tuned to 940 nm |
| Kingbright APED3528F3C | IR emitter (LED) | SMD, dome lens | 940 nm, 35° beam — reduces sideways leakage into the adjacent photodiode |
| SunLED XZMYK53W-8 | IR emitter, thinner alternative | 0603/0805 SMD | Lower power, for the thinnest frame sections |
| 74HC4051 / CD4051B | 8-channel analogue multiplexer | TSSOP/QFN SMD | Routes multiple photodiodes into one ADC pin |
| 74HC595 | Shift register | SMD | Sequences LED firing from a few control pins |

## 4.3 Orientation & cross-talk control

- Tilt each LED/photodiode pair toward the eye in a shallow “V”: LED ≈+15°, photodiode ≈-15°, so the sensed reflection comes from the eyeball surface, not a direct LED-to-photodiode bounce.

- Outer nodes (temple side) tilt steeper, ≈20–25°; inner nodes (nose-bridge side) tilt shallower, ≈10–15°, so they don’t flood with reflections off the skin of the nose.

- A small physical divider rib between each LED/photodiode pair blocks direct surface light leakage.

- For binocular operation, mirror the geometry across the bridge — left and right assemblies are separate, independently tuned zones, which simplifies calibration and cross-talk control versus one shared central array.

## 4.4 Firmware: time-division multiplexing

Do not run all LEDs simultaneously — cross-talk between channels, and between the two eyes, makes the readings unusable. Strobe one LED, read its matching photodiode, then move to the next node, cycling through all nodes for both eyes. Photodiodes in this class respond in well under 100 ns, so a full multi-node cycle across both eyes can complete in under 1 ms, putting the theoretical sampling rate into the kHz range — confirm the achievable rate once the real component count and microcontroller ADC budget are fixed.

## 4.5 Derived outputs

- Left-eye gaze vector, right-eye gaze vector.

- Blink detection, eye openness.

- Approximate pupil size.

- Vergence (from the angle between the two gaze vectors) → fixation distance, with no extra eye-side depth hardware.

- Approximate eye-to-lens distance for calibration (roughly 12–17 mm range).

## 4.6 Fusion with the front camera stream

Vergence alone estimates how far away the fixation point is; pairing the gaze rays with the 3D scene map derived from the front camera stream lets the system identify which actual object is being looked at, not just how far away it sits. Per this revision, that fusion step is assumed to run on the mainboard’s ML pipeline alongside object detection, rather than on the bridge module — so the bridge needs to send both the gaze-array outputs (Section 4.5) and the front camera stream (Section 3) to the mainboard.

## 4.7 Accuracy trade-off

Flat photodiode arrays trade spatial resolution and calibration simplicity for a large win in power, thickness, and privacy versus inward-facing cameras. They suit interaction, accessibility, and attention-tracking use cases well. If the product’s target is sub-degree gaze accuracy for scientific or high-end AR applications, inward-facing IR cameras remain the more proven approach — make this trade-off explicitly and document it before committing to the photodiode approach in tooling or production.

# 5. Industrial Design Direction (from reference renders)

- CNC-machined aluminum housing, matte black anodized finish.

- Front face: two camera lenses, illumination/status elements, a low-profile horizontal bridge bar.

- Rear face: two independent eye zones, each a flush smoked IR-transparent window with the photodiode/LED matrix hidden behind it, separated by a narrow central electronics section.

- No exposed screws, no visible wiring, no labels, logos, or text on the housing.

- Target fit-and-finish benchmark from the design brief: premium consumer-electronics tier rather than a visible “sensor panel.”

# 6. Companion Mainboard

Carried over from v0.2, with one significant addition: the mainboard now hosts the live ML image-processing and object-detection pipeline.

- Radio: needs reassessment — see Section 9. The original Nordic nRF52840-class radio was sized for small coordinate packets, not a continuous video stream.

- Antenna: SMA or U.FL connector with a flexible 2.4 GHz (or higher-band, depending on the final radio choice) antenna.

- Battery: standard LiPo, ≈1000 mAh class — size and weight are not constrained here, and now needs to cover the ML pipeline’s compute load in addition to radio reception.

- Firmware/software role: receive the bridge module’s live camera stream and gaze-array outputs, run image processing and object detection in real time, fuse the result with the gaze vectors, and process it — with no image data written to persistent storage anywhere in the pipeline, per the stated design principle.

Confirm where the ML inference actually runs (this document assumes the mainboard — see Section 1) and what its own power/thermal draw looks like once a model and hardware target are chosen; that is a separate budget from the bridge module’s, but it will affect mainboard battery life and possibly its size/cooling.

# 7. Power Budget, Battery Life & Fast-Charge Calculations

## 7.1 What changed

The front camera pair now streams live, continuously, to the mainboard’s ML pipeline instead of the bridge module fusing everything on-board. That single change is what dominates this recalculation: it removes the on-board vision processor’s draw, but replaces the original lightweight coordinate-packet radio link with a continuous video link, which costs far more power than the processor it replaces.

These are component-class engineering estimates, not measured figures — final numbers depend on the camera, radio, and codec parts ultimately chosen (Sections 3, 9, 10). The rear gaze array (Section 4) is assumed unchanged and is not part of what is driving this increase.

## 7.2 Estimated power draw at full continuous streaming

| Subsystem | Est. avg draw | Basis |
| --- | --- | --- |
| Front RGB/IR camera pair (2×) | 200 mW | ≈100 mW each, small global-shutter sensors, continuous streaming |
| Front IR illumination (low-light assist) | 30 mW | Blended average duty |
| Rear IR gaze array (photodiodes + LEDs, TDM-scanned) | 15 mW | Unchanged from the prior estimate — this part genuinely is low-power |
| IMU | 2 mW |  |
| On-board MCU / sensor control | 15 mW | Lightweight now that ML inference has moved to the mainboard |
| Wireless radio, continuous video stream | 200 mW | BLE/ESB cannot carry this; modelled as a Wi-Fi-class radio — see Section 9 |
| TOTAL (100% active-streaming duty) | ≈ 462 mW |  |

At LTO’s nominal cell voltage (≈2.4 V): 462 mW ÷ 2.4 V ≈ 192 mA average — roughly 125× the original 1.5 mA estimate from the pre-streaming design.

## 7.3 Battery life at full continuous streaming

| Candidate cell size | Runtime at ≈192 mA | Note |
| --- | --- | --- |
| 50 mAh | ≈16 minutes | Smallest realistic micro-cell class |
| 100 mAh | ≈31 minutes |  |
| 150 mAh | ≈47 minutes |  |
| 250 mAh | ≈1 hour 18 minutes | Likely already too large for an “incredibly small” nose-bridge module |

## 7.4 Battery life at partial duty cycle

If the bridge module is not always actively streaming — e.g. it drops to a low-power idle state (estimated ≈25 mW: rear gaze array running, minimal MCU/IMU draw, low-power radio beacon) when not in active use — average draw improves, but the 48-hour target still does not close:

| Active duty cycle | Blended avg power | Blended avg current | Capacity needed for 48 h |
| --- | --- | --- | --- |
| 100% | 462 mW | ≈192 mA | ≈9,220 mAh |
| 50% | ≈244 mW | ≈102 mA | ≈4,870 mAh |
| 25% | ≈134 mW | ≈56 mA | ≈2,680 mAh |
| 10% | ≈69 mW | ≈29 mA | ≈1,370 mAh |

HEADLINE FINDING: even at a fairly light 10% active-streaming duty cycle, the original 48-hour target would need on the order of 1,300+ mAh of capacity — far beyond what a nose-bridge-sized module can physically hold. Continuous live video streaming for ML object detection and the original “incredibly small, 48-hour” target are not simultaneously achievable in this architecture. Something has to change.

## 7.5 Practical paths forward (not mutually exclusive)

- Deliberately shrink the active-streaming duty cycle — e.g. event- or gesture-triggered capture rather than always-on streaming.

- Move more of the camera/radio hardware to the mainboard side, keeping the bridge module limited to optics and a thin, lower-power link.

- Compress or down-sample the video stream more aggressively, and/or reduce frame rate, to lower the radio’s draw.

- Accept a much shorter standalone runtime (tens of minutes to roughly an hour at realistic cell sizes) and lean on the charging case’s fast top-up for the user experience — see Section 7.6.

## 7.6 Fast-charge feasibility (10-minute target)

The good news: the 10-minute fast-charge target stays technically achievable with LTO chemistry at any of the cell sizes above — it is the much shorter runtime, not the charge time, that breaks under this architecture.

| Cell size | Charge current for 10-min full charge | C-rate | Feasibility note |
| --- | --- | --- | --- |
| 50 mAh | 300 mA | 6C | Low absolute current — easiest to validate against the charge IC and contacts |
| 100 mAh | 600 mA | 6C | Check against the BQ25120A’s maximum rated output current before locking this in |
| 150 mAh | 900 mA | 6C | Likely exceeds many ultra-small charge ICs’ rated output; may need a different charge-management part |
| 250 mAh | 1,500 mA | 6C | Almost certainly needs a larger charge IC and heavier-duty pogo-pin contacts |

- Confirm the chosen charge-management IC’s maximum rated output current against whichever cell size is finally selected — do not assume the BQ25120A scales to all of the above without checking its datasheet.

- Confirm the pogo-pin contacts’ current-carrying rating at the higher end of this range (900–1,500 mA) — tiny spring contacts have real current limits before resistive heating becomes a problem.

- The charging case’s ≈1,000 mAh reservoir LiPo discharging at 0.3–1.5 C to deliver these currents is comfortably within normal LiPo capability — this side of the system is not the bottleneck.

# 8. Charging Case

Unchanged in concept: internal Qi receiver coil for ambient desk top-up, an internal reservoir LiPo (≈1000 mAh) that buffers energy, and a spring-loaded pogo-pin dock that delivers a fast charge to the bridge module’s battery once contact is verified. See Section 7.6 for the resolved charge-current figures and the two component checks (charge IC rating, pogo-pin current rating) still needed before finalizing a cell size.

# 9. Wireless Link Layer

RE-OPENED BY THIS REVISION: the BLE/Enhanced ShockBurst link below was sized for small coordinate packets, not continuous video. It almost certainly cannot reliably carry two live camera streams, even compressed. Evaluate a Wi-Fi-class (802.11) or UWB radio for the bridge-to-mainboard link before locking in any of the figures below, and re-derive the indoor-range table once a technology is chosen, since range and obstacle behaviour differ meaningfully between BLE, Wi-Fi, and UWB.

## 9.1 Indoor range expectations (estimates from the pre-streaming design; need re-deriving for the final radio choice)

| Obstacle type | Signal impact | Estimated max distance |
| --- | --- | --- |
| Open-plan / same room | None | 30+ m |
| Drywall / stud walls | Low | 20–25 m (3–4 rooms) |
| Brick / concrete walls | Medium | 10–15 m (1–2 dense walls) |
| Concrete floors / ceilings | High | 1–2 floors maximum |

## 9.2 Dynamic TX power loop

Bridge module sends a packet → Mainboard measures RSSI → Mainboard returns an acknowledgement containing that RSSI → Bridge module tunes its next transmit power: strong signal drops power to save battery; weak signal ramps power up for range.

## 9.3 Security

- AES-128 hardware-accelerated encryption on the radio link, or the chosen radio’s native equivalent.

- Do not hardcode the encryption key in firmware. Generate session keys via the chip’s on-board TRNG, or add a dedicated secure-element IC (e.g. ATECC608A) on both boards if a higher security bar is needed.

Re-confirm the specific cipher suite once the final radio technology (Section 9, intro) is chosen — some Wi-Fi-class radios offer their own native encryption that may simplify or change this.

# 10. Bill of Materials (working list)

| Component | Role | Package / form | Notes |
| --- | --- | --- | --- |
| Stereo RGB/IR camera pair | Front scene capture, streamed live | TBD | Open item — see Section 3 |
| IMU (+ optional ToF) | Head motion / scene-mapping support | TBD | Open item |
| Vishay VEMD2704 | Gaze-array photodiode | 2.3×2.3 mm SMD | Daylight-blocking filter, 940 nm tuned |
| Kingbright APED3528F3C | Gaze-array IR emitter | SMD, dome lens | 940 nm, 35° beam |
| 74HC4051 / CD4051B | Photodiode multiplexer | TSSOP/QFN SMD | One per eye, or shared with care |
| 74HC595 | LED shift register | SMD |  |
| Wi-Fi-class or UWB radio module | Bridge-to-mainboard video link | TBD | Replaces the nRF52 BLE/ESB radio for this link — see Section 9 |
| LTO micro pin/button cell | Bridge module battery | 50–150 mAh class (TBD) | Size to be chosen per Section 7.3–7.4 trade-off |
| TI BQ25120A (or larger alternative) | Bridge module charge management IC | 2.5×2.5 mm or larger | Confirm max rated current against the chosen cell size — Section 7.6 |
| ML-capable SoC/module | Mainboard image processing & object detection | TBD | New item — hosts the ML pipeline per Section 6 |
| Reservoir LiPo | Case buffer battery | ≈1000 mAh |  |
| Qi receiver coil | Case ambient charging | TBD |  |
| ATECC608A (optional) | Secure key storage | Small SMD | Add only if higher security bar required |

# 11. Open Engineering Questions

- Confirm the streaming architecture assumption (Section 1): does “ir images” refer to the front scene camera’s low-light/IR mode, or does the rear eye-tracking also need to become camera-based? This materially changes Section 4 and the Section 7 numbers.

- Confirm where the ML pipeline actually runs (assumed: the mainboard) and its own power/thermal budget once a model and hardware target are chosen.

- Resolve the battery-size-vs-duty-cycle trade-off in Section 7.4–7.5 — pick a target runtime and duty-cycle behaviour (always-on vs. event-triggered) before locking the cell size.

- Choose the bridge-to-mainboard radio technology (Wi-Fi-class vs. UWB) and video codec/compression approach — drives both the power figure in Section 7.2 and the range table in Section 9.1.

- Confirm the charge-management IC’s and pogo-pin contacts’ current ratings against whichever cell size is finally chosen (Section 7.6).

- Final photodiode/LED count per eye (16 vs. 32 elements; 4-node vs. full matrix) not yet locked — affects accuracy and firmware complexity.

- Mechanical integration of the rear smoked window with prescription-lens curvature/fit — not yet addressed.

# 12. Next Steps

- Get explicit confirmation on the Section 1 assumption before doing further design work on the rear gaze array or the front camera stream.

- Pick a target duty cycle and cell size from Section 7.4, then validate the charge IC and pogo-pin current ratings against it.

- Prototype the bridge-to-mainboard video link on a Wi-Fi-class or UWB dev kit to get a real power measurement in place of the Section 7.2 estimate.

- Bench-test a single-eye photodiode array (4-node differential) on a flex PCB prototype before committing to a binocular layout.

- Validate TDM firmware cross-talk rejection across both eyes simultaneously.

- Re-confirm LTO cell and charge-rate assumptions once real current draw is known.