# Candour

Engineering Specification — Working Draft

| Field | Value |
| --- | --- |
| Document title | Candour — Engineering Specification |
| Version | v0.2 (working draft — supersedes v0.1, “ReArm Snap’n’Track”) |
| Status | Draft — major architecture update (front stereo cameras + rear binocular IR gaze array); not bench-validated |
| Owner | [Your name] |
| Last updated | 26 June 2026 |
| Distribution | Internal / maker use only — not for external release |

# 1. Product Overview

Candour is a smart-eyewear bridge module for prescription or cosmetic glasses that adds two capabilities: outward-facing stereo depth sensing of the environment, and inward-facing binocular gaze tracking of the wearer’s eyes — fused together to determine exactly what the wearer is looking at and how far away it is. A separate, less space-constrained companion mainboard (worn or carried nearby) handles heavier compute, logging, and battery duty.

- Bridge module integrates cleanly into the glasses’ nose bridge with a premium, CNC-machined finish — no exposed screws or visible wiring.

- Inward gaze sensing must not use inward-facing cameras (privacy, power, and thickness) — use a flat IR photodiode array instead.

- Outward sensing uses stereo RGB cameras to build a 3D map of the scene.

- Gaze vectors (inward) combine with the 3D scene map (outward) to identify exactly what the user is looking at and how far away it is — no extra depth camera is needed on the eye side; vergence between the two gaze vectors gives fixation depth on its own.

- Wireless link to the mainboard keeps the original in-home range target (≈15–30 m); the payload should stay lightweight — processed vectors and events, not raw imagery.

- All communication encrypted end-to-end.

SCOPE CHANGE: adding outward-facing stereo cameras and an on-board AI vision processor is a substantial change from the original ultra-low-power, ultra-tiny “nose-bridge sensor” concept. The original 1.5 mA / 72 mAh power budget (carried over from v0.1) was sized for a dormant IR sensor and radio only — it almost certainly will not cover active stereo cameras plus a vision coprocessor. The entire power budget is open for rework; see Section 7.

# 2. System Architecture

| Component | Role | Notes |
| --- | --- | --- |
| Bridge Module (on the glasses) | Outward stereo depth sensing + inward binocular gaze tracking + on-board fusion compute | Replaces the original “Satellite” node; now houses meaningfully more silicon than originally scoped |
| Companion Mainboard (carried / placed) | Receives processed outputs; heavier logging/compute; larger battery | Role unchanged from v0.1 |
| Charging Case (accessory) | Reservoir battery, Qi receiver coil, pogo-pin dock | Concept unchanged; charge profile needs re-deriving once real bridge power draw is known |

Data flow:

The front stereo cameras build a 3D map of the scene. The rear photodiode arrays estimate each eye’s gaze vector, blink, pupil size, and eye openness, plus vergence (the convergence angle between the two gaze vectors, which gives fixation distance with no extra depth hardware on the eye side). The bridge’s on-board vision processor fuses the gaze rays with the 3D scene map to determine the object being looked at and its distance. Only this fused, compact result — not raw video and not raw photodiode traces — is intended to be sent over the wireless link to the mainboard, consistent with the original “coordinates, not video” design principle.

# 3. Bridge Module — Front-Facing Stack (outward, away from the eyes)

Per the reference render: a single horizontal bridge housing spans the nose, with two RGB camera lenses, illumination elements, and a status indicator visible on the outward face.

- Stereo RGB camera pair — for 3D scene mapping / SLAM.

- Optional IR or visible illumination — for low-light scene capture (visible in the render; exact part TBD).

- IMU, and optionally a ToF sensor — for head motion/orientation, supporting the scene-mapping pipeline.

- On-board AI vision / fusion processor — runs scene reconstruction and gaze-ray fusion locally rather than streaming raw camera data off the device.

- Status LED — visible in the render; recommend reserving this as an active-sensing/recording indicator (see the compliance specification’s bystander-privacy section).

OPEN ITEM: exact camera modules, illumination part numbers, IMU/ToF part, and vision-processor silicon are not yet specified. This is the single highest-priority item to scope next, since it drives the entire power and thermal budget.

# 4. Bridge Module — Rear-Facing Stack: the Gaze Array (inward, toward the eyes)

This replaces the previously open “gaze sensor” item from the original spec. Approach: a flat IR photodiode array per eye, with no inward-facing cameras, using differential reflectance off the sclera, iris, pupil, and eyelid to infer gaze (sometimes called photoscleral oculography).

## 4.1 Per-eye sensor geometry

- 16–32 IR photodiodes per eye, arranged as a 4×4 or 5×5 matrix, or as a simpler 4-node differential quadrant (top / bottom / left / right) around the eye perimeter.

- 2–4 IR LEDs per eye, 940 nm — invisible to the human retina, avoiding glare or distraction.

- Both sit behind a flush, smoked IR-transparent window so nothing is visible from outside the lens — matching the reference render’s intent of looking like sealed consumer electronics rather than a visible sensor grid.

## 4.2 Candidate components

| Component | Role | Package | Notes |
| --- | --- | --- | --- |
| Vishay VEMD2704 | IR photodiode (receiver) | 2.3×2.3 mm leadless SMD | Daylight-blocking filter, 830–950 nm, ≈70 ns response — fast enough for saccades |
| Everlight PD15-22C/TR8 | IR photodiode, lower-cost alternative | 1206 SMD | Tuned to 940 nm |
| Kingbright APED3528F3C | IR emitter (LED) | SMD, dome lens | 940 nm, 35° beam — reduces sideways leakage into the adjacent photodiode |
| SunLED XZMYK53W-8 | IR emitter, thinner alternative | 0603/0805 SMD | Lower power, for the thinnest frame sections |
| 74HC4051 / CD4051B | 8-channel analogue multiplexer | TSSOP/QFN SMD | Routes multiple photodiodes into one ADC pin |
| 74HC595 | Shift register | SMD | Sequences LED firing from a few control pins |

## 4.3 Orientation & cross-talk control

- Tilt each LED/photodiode pair toward the eye in a shallow “V”: LED ≈+15°, photodiode ≈-15°, so the sensed reflection comes from the eyeball surface, not a direct LED-to-photodiode bounce.

- Outer nodes (temple side) tilt steeper, ≈20–25°; inner nodes (nose-bridge side) tilt shallower, ≈10–15°, so they don’t flood with reflections off the skin of the nose.

- A small physical divider rib between each LED/photodiode pair blocks direct surface light leakage.

- For binocular operation, mirror the geometry across the bridge — left and right assemblies are separate, independently tuned zones, which simplifies calibration and cross-talk control versus one shared central array.

## 4.4 Firmware: time-division multiplexing

Do not run all LEDs simultaneously — cross-talk between channels, and between the two eyes, makes the readings unusable. Strobe one LED, read its matching photodiode, then move to the next node, cycling through all nodes for both eyes. Photodiodes in this class respond in well under 100 ns, so a full multi-node cycle across both eyes can complete in under 1 ms, putting the theoretical sampling rate into the kHz range — confirm the achievable rate once the real component count and microcontroller ADC budget are fixed.

## 4.5 Derived outputs

- Left-eye gaze vector, right-eye gaze vector.

- Blink detection, eye openness.

- Approximate pupil size.

- Vergence (from the angle between the two gaze vectors) → fixation distance, with no extra eye-side depth hardware.

- Approximate eye-to-lens distance for calibration (roughly 12–17 mm range).

## 4.6 Fusion with the front stereo system

Vergence alone estimates how far away the fixation point is; pairing the gaze rays with the front stereo cameras’ 3D scene map lets the system identify which actual object is being looked at, not just how far away it sits. This fused result is the natural payload to send to the mainboard.

## 4.7 Accuracy trade-off

Flat photodiode arrays trade spatial resolution and calibration simplicity for a large win in power, thickness, and privacy versus inward-facing cameras. They suit interaction, accessibility, and attention-tracking use cases well. If the product’s target is sub-degree gaze accuracy for scientific or high-end AR applications, inward-facing IR cameras remain the more proven approach — make this trade-off explicitly and document it before committing to the photodiode approach in tooling or production.

# 5. Industrial Design Direction (from reference renders)

- CNC-machined aluminum housing, matte black anodized finish.

- Front face: two camera lenses, illumination/status elements, a low-profile horizontal bridge bar.

- Rear face: two independent eye zones, each a flush smoked IR-transparent window with the photodiode/LED matrix hidden behind it, separated by a narrow central electronics section.

- No exposed screws, no visible wiring, no labels, logos, or text on the housing.

- Target fit-and-finish benchmark from the design brief: premium consumer-electronics tier rather than a visible “sensor panel.”

# 6. Companion Mainboard

Carried over from v0.1, pending the bandwidth re-confirmation noted below.

- Radio: Nordic nRF52840-class module/dongle, matching the bridge module’s 2.4 GHz protocol.

- Antenna: SMA or U.FL connector with a flexible 2.4 GHz patch/“rubber duck” antenna — improves margin when the bridge signal is weak or the mainboard is laid flat.

- Battery: standard LiPo, ≈1000 mAh class — size and weight are not constrained here.

- Firmware role: receive and decrypt the fused gaze/scene output, log/process it, and run the RSSI feedback loop that drives the bridge module’s transmit-power adjustment.

Confirm the wireless payload (Section 4.6) stays small enough for the existing low-power 2.4 GHz link. If any compressed imagery or point-cloud data ever needs to leave the bridge module, the link budget and protocol choice need to be revisited.

# 7. Power Budget — OPEN, NEEDS FULL REWORK

This is the top-priority open item in this revision.

The v0.1 estimate (1.5 mA average → 72 mAh for 48 hours) assumed a dormant IR sensor and a low-duty-cycle radio only. It does not account for:

- Continuously running stereo RGB cameras.

- An on-board AI vision/fusion processor.

- 8+ time-division-multiplexed IR LED/photodiode channels per eye, run at up to kHz rates.

Re-derive the power budget once the camera, vision-processor, and final gaze-array component counts are chosen, and re-check whether the 48-hour runtime / 10-minute charge targets — and the LTO battery chemistry chosen to support them — are still realistic at the new current draw.

# 8. Charging Case

Unchanged in concept from v0.1: internal Qi receiver coil for ambient desk top-up, an internal reservoir LiPo (≈1000 mAh) that buffers energy, and a spring-loaded pogo-pin dock that delivers a fast charge to the bridge module’s battery once contact is verified. The charge profile (rate, duration) depends entirely on the Section 7 rework and should not be finalized until that is resolved.

# 9. Wireless Link Layer

## 9.1 Indoor range expectations (estimates, not yet measured)

| Obstacle type | Signal impact | Estimated max distance |
| --- | --- | --- |
| Open-plan / same room | None | 30+ m |
| Drywall / stud walls | Low | 20–25 m (3–4 rooms) |
| Brick / concrete walls | Medium | 10–15 m (1–2 dense walls) |
| Concrete floors / ceilings | High | 1–2 floors maximum |

## 9.2 Dynamic TX power loop

Bridge module sends a packet → Mainboard measures RSSI → Mainboard returns an acknowledgement containing that RSSI → Bridge module tunes its next transmit power: strong signal drops power to save battery; weak signal ramps power up for range.

## 9.3 Security

- AES-128 hardware-accelerated encryption on the radio link.

- Do not hardcode the encryption key in firmware. Generate session keys via the chip’s on-board TRNG, or add a dedicated secure-element IC (e.g. ATECC608A) on both boards if a higher security bar is needed.

# 10. Bill of Materials (working list)

| Component | Role | Package / form | Notes |
| --- | --- | --- | --- |
| Stereo RGB camera pair | Front scene capture | TBD | Open item — see Section 3 |
| IMU (+ optional ToF) | Head motion / scene-mapping support | TBD | Open item |
| AI vision / fusion processor | On-board scene + gaze fusion | TBD | Drives the whole power budget — top priority to select |
| Vishay VEMD2704 | Gaze-array photodiode | 2.3×2.3 mm SMD | Daylight-blocking filter, 940 nm tuned |
| Kingbright APED3528F3C | Gaze-array IR emitter | SMD, dome lens | 940 nm, 35° beam |
| 74HC4051 / CD4051B | Photodiode multiplexer | TSSOP/QFN SMD | One per eye, or shared with care |
| 74HC595 | LED shift register | SMD |  |
| Nordic nRF52840 module/dongle | Mainboard radio | Module or dev kit | Matches bridge module protocol |
| SMA/U.FL + patch antenna | Mainboard antenna | Flexible external |  |
| Reservoir LiPo | Case buffer battery | ≈1000 mAh |  |
| Qi receiver coil | Case ambient charging | TBD |  |
| LTO micro pin/button cell | Bridge module battery | TBD capacity | Capacity to be re-derived per Section 7 |
| TI BQ25120A | Bridge module charge management IC | 2.5×2.5 mm | Charge profile to be re-derived per Section 7 |
| ATECC608A (optional) | Secure key storage | Small SMD | Add only if higher security bar required |

# 11. Open Engineering Questions

- Power budget must be fully re-derived (Section 7) — top priority.

- Camera modules, illumination, IMU/ToF, and vision-processor silicon not yet chosen.

- Final photodiode/LED count per eye (16 vs. 32 elements; 4-node vs. full matrix) not yet locked — affects accuracy and firmware complexity.

- Achievable real-world sampling rate once the ADC/mux budget is fixed.

- Confirm the wireless payload definition (Section 6) and re-check the link budget once it’s final.

- Mechanical integration of the rear smoked window with prescription-lens curvature/fit — not yet addressed.

- Protocol choice (BLE/ESB) should be re-confirmed once bandwidth needs from the new front stack are known.

# 12. Next Steps

- Lock the front-stack BOM (camera/IMU/vision processor) and re-run the power budget.

- Bench-test a single-eye photodiode array (4-node differential) on a flex PCB prototype before committing to a binocular layout.

- Validate TDM firmware cross-talk rejection across both eyes simultaneously.

- Re-confirm LTO cell and charge-rate assumptions once real current draw is known.