# ReArm Candour

Engineering Specification — Working Draft

| Field | Value |
| --- | --- |
| Document title | ReArm Candour — Engineering Specification |
| Version | v0.1 (working draft) |
| Status | Draft — architecture exploration; not bench-validated |
| Owner | [Your name] |
| Last updated | 19 June 2026 |
| Distribution | Internal / maker use only — not for external release |

# 1. Product Overview

ReArm Candour is a two-piece add-on system for prescription or cosmetic eyeglasses that adds gaze (eye-tracking) capability without materially changing the size, weight, or balance of the glasses. A tiny sensor module sits on the nose bridge and streams compressed gaze-coordinate data wirelessly to a separate, less space-constrained mainboard that the user carries or sets down nearby.

Design goals (from requirements gathering):

- Nose-bridge module must be near-invisible: minimal mass, no forward pull on the frame.

- Wireless link only needs to cover “across the house” distances (roughly 15–30 m, degraded by walls).

- Link must keep working whether the mainboard is on the user’s body or set down stationary.

- Nose-bridge battery: 48-hour runtime per charge, full recharge in ≤10 minutes.

- Communication must be encrypted end-to-end.

# 2. System Architecture

The system has two active nodes plus one charging accessory:

| Node | Role | Key constraint |
| --- | --- | --- |
| Satellite (nose bridge) | IR/optical gaze sensing + on-chip coordinate extraction + radio | Ultra-small, ultra-light, low heat |
| Mainboard (carried or set down) | Radio + logging/compute + larger battery | Must keep the link reliable whether worn or stationary |
| Charging case (accessory) | Houses reservoir battery, Qi receiver coil, and pogo-pin dock | Delivers the 10-minute fast top-up and ambient desk charging |

Data flow:

Satellite captures gaze → computes X/Y coordinates on-chip → encrypts → transmits coordinate packet → Mainboard receives and decrypts, logs/processes, and returns an acknowledgement carrying RSSI → Satellite adjusts its next transmit power accordingly.

# 3. Satellite Node (Nose Bridge) — Detailed Spec

## 3.1 Compute + Radio

- Primary candidate: Nordic Semiconductor nRF52832 or nRF52811, WLCSP package (≈ 2.5 × 2.5 mm) — combines a BLE-class radio and a Cortex-M4 MCU in one die-scale package.

- Protocol: not standard BLE GATT — use Nordic Enhanced ShockBurst (ESB), a lightweight proprietary 2.4 GHz packet protocol, to avoid BLE connection-interval latency.

- Hardware-accelerated AES-128 encryption block on-chip.

- Prototyping substitutes before committing to the WLCSP layout: Holyiot 18010 module (7 × 9 mm, same die) or a Seeed XIAO nRF52840 dev board, to bench-test the radio/firmware stack first.

## 3.2 Gaze Sensing — OPEN ITEM

Exact sensor not yet chosen. Hard requirement: the module must output X/Y gaze coordinates on-chip/in-module — raw video must never be transmitted wirelessly (bandwidth, battery, and privacy all argue against it). Candidates to evaluate: a miniature IR camera with on-board image processing, or a photodiode-array reflectance tracker.

## 3.3 Antenna

- Johanson Technology 2450AT18A100E ceramic chip antenna, 3.2 × 1.6 mm, omnidirectional pattern — keeps the link alive regardless of head angle.

- Must clear the ground-plane directly beneath the antenna per the manufacturer’s datasheet — confirm in layout review.

## 3.4 Power Budget

Target average current draw: ≈ 1.5 mA (low-power radio + low-power gaze sensing).

1.5 mA × 48 h = 72 mAh required capacity.

This is an estimate, not yet bench-validated. Confirm once the gaze sensor is selected — sensor draw may end up dominating the budget.

## 3.5 Battery

- Lithium Titanate (LTO) micro pin/button cell, ≈72 mAh class.

- Rationale: standard LiPo is limited to roughly 1C safe charging (≈ 1 hour for a full charge). Reaching a 10-minute charge needs about 6C, which risks degrading or overheating standard LiPo chemistry. LTO tolerates 6–20C charge rates safely and is rated for 10,000–20,000+ cycles.

- Charge controller: Texas Instruments BQ25120A (2.5 × 2.5 mm), I²C-programmable current control, sized for microbatteries.

## 3.6 Charging Interface

No bulky USB port on the nose bridge. Use spring-loaded pogo-pin contacts built into the nose-pad structure, mating with a cradle in the storage case when the glasses are folded and stored.

## 3.7 PCB & Assembly Notes

- WLCSP/QFN-class parts require a 4-layer PCB, 0402-class passives, and reflow assembly (hot-air or hot-plate) — not realistically hand-solderable at final size.

- Bench-validate the radio/firmware/protocol stack on a breakout module (see 3.1) before committing to the final miniature layout.

# 4. Mainboard Node — Detailed Spec

## 4.1 Compute + Radio

Must speak the same 2.4 GHz ESB/BLE protocol as the satellite. A Nordic nRF52840 (dev kit, dongle, or module) is the natural pairing partner since it shares the radio family with the satellite chip.

Earlier scoping discussed an ESP32-S3 + Semtech SX1262 LoRa pairing — that combination targets kilometre-scale outdoor links and is NOT compatible with the nRF52 ESB protocol. Lock in BLE/ESB end-to-end before further layout work; mixing a LoRa radio on one node with a BLE radio on the other won’t talk without a protocol bridge.

## 4.2 Antenna

SMA or U.FL connector with a flexible 2.4 GHz “rubber duck” or patch antenna — not a flat chip antenna. The larger antenna improves margin when the satellite signal is weak, and helps when the mainboard is laid flat on a surface.

## 4.3 Power

Standard LiPo, ≈1000 mAh class — size and weight are not constrained on this node.

## 4.4 Firmware Role

Receive and decrypt gaze packets, log/process data, and run the RSSI feedback loop: measure RSSI per received packet, return it in the acknowledgement, and let the satellite tune its own transmit power accordingly.

# 5. Wireless Link Layer

## 5.1 Indoor Range Expectations (estimates, not yet measured)

| Obstacle type | Signal impact | Estimated max distance |
| --- | --- | --- |
| Open-plan / same room | None | 30+ m |
| Drywall / stud walls | Low | 20–25 m (3–4 rooms) |
| Brick / concrete walls | Medium | 10–15 m (1–2 dense walls) |
| Concrete floors / ceilings | High | 1–2 floors maximum |

## 5.2 Dynamic TX Power Loop

Satellite sends a gaze packet → Mainboard measures RSSI → Mainboard returns an acknowledgement containing that RSSI → Satellite tunes its next transmit power: strong signal (mainboard nearby, e.g. in a pocket) drops power to roughly –20 dBm to save battery; weak signal (mainboard left across the house) ramps power up to +4/+8 dBm for range.

## 5.3 Security

- AES-128 hardware-accelerated encryption on the nRF52 radio link.

- Do not hardcode the encryption key in firmware source. Generate session keys using the chip’s on-board TRNG, or add a dedicated secure-element IC (e.g. ATECC608A) on both boards for key storage/exchange if a higher security bar is needed later.

# 6. Charging Case (Accessory)

- Internal Qi wireless receiver coil — ambient top-up whenever the case is set on any Qi charging pad or desk dock.

- Internal reservoir LiPo, ≈1000 mAh — buffers energy so the case doesn’t need to be plugged in for the satellite’s fast charge.

- Spring-loaded pogo-pin dock aligns with the nose-pad contacts when the glasses fold into the case; a step-up/down converter dumps high current into the LTO cell once contact is verified, completing the ≤10-minute charge.

# 7. Bill of Materials (working list)

| Component | Role | Package / form | Notes |
| --- | --- | --- | --- |
| Nordic nRF52832 / nRF52811 | Satellite MCU + radio | WLCSP ≈2.5×2.5 mm | Final form factor; needs reflow assembly |
| Holyiot 18010 module | Bench prototype of above | 7×9 mm module | For early firmware bring-up |
| Seeed XIAO nRF52840 | Bench prototype, mainboard radio | 21×17.8 mm dev board | ≈£7–£10 |
| Johanson 2450AT18A100E | Satellite antenna | 3.2×1.6 mm ceramic chip | Confirm ground-plane clearance |
| TI BQ25120A | Satellite charge mgmt IC | 2.5×2.5 mm | I²C programmable, microbattery-rated |
| LTO micro pin/button cell | Satellite battery | ≈72 mAh class | Source/vendor TBD |
| Qi receiver coil | Case ambient charging | TBD | Spec to be finalized |
| Reservoir LiPo | Case buffer battery | ≈1000 mAh |  |
| nRF52840 dongle / module | Mainboard MCU + radio | Dev kit or module |  |
| SMA/U.FL + patch antenna | Mainboard antenna | Flexible external |  |
| ATECC608A (optional) | Secure key storage, both boards | Small SMD | Add only if higher security bar required |

# 8. Open Engineering Questions

- Gaze sensor: IR camera vs. photodiode array vs. dedicated eye-tracking IC — this choice significantly affects the power budget.

- Confirm the protocol end-to-end as BLE/ESB only; retire the earlier LoRa option, which was scoped for a kilometre-range use case, not this in-house one.

- Exact LTO cell part/vendor and confirmed operating voltage, to finalize the BQ25120A charge profile.

- EDA tool for layout (KiCad / Altium / EasyEDA) — affects which footprint libraries to source.

- Real-world RF testing: the range table in Section 5.1 is an estimate; validate with bench prototypes through the actual target home’s wall types before locking antenna and power settings.

- Mainboard housing/form factor — not yet defined.

- Mechanical attachment of the satellite to the nose bridge (clip-on vs. adhesive vs. integrated nose pad) — not yet defined.

# 9. Next Steps

- Bench-validate the radio link and RSSI loop using XIAO dev boards before laying out either custom PCB.

- Select and characterize the gaze sensor; re-run the power budget once real current draw is known.

- Source and bench-test an LTO cell with the BQ25120A charge profile to confirm the 10-minute claim.

- Lock the antenna footprint and ground-plane clearance against the antenna vendor’s reference layout.